World Components Network Service Ltd

WCN1-00D0SD-A11S

SPECIFICATION

| WCN |  |  | CUSTOMER |
| --- | --- | --- | --- |
| Prepared by | Checked by | Approved by |  |
| NOTE: |  |  |  |

REVISION:  02

1/3

World Components Network Service Ltd

Outer Dimension:

Ф0.60±0.1

- Note: Unless otherwise stated , The tolerance is±0.25 mm.

- Circuit Diagram:

■Pin Connection:

| PIN NO. | CONNECTION | PIN NO. | CONNECTION |
| --- | --- | --- | --- |
| 1 | Common Anode | 6 | Cathode B |
| 2 | Cathode E | 7 | Cathode A |
| 3 | Cathode D | 8 | Common Anode |
| 4 | Cathode C | 9 | Cathode F |
| 5 | Cathode DP | 10 | Cathode G |

2/3

WCN1-00D0SD-A11S

World Components Network Service Ltd

■ Features:

.  High Reliability

.  Color: Super Red

.  Low Power Requirement

.  Easy Assembly

- Description:

.  Single Digit LED Display

.  Digit Height:101.2mm(4.0〞)

.  Black Face And Encapsulation With Milky Segment

■Absolute Maximum Rating (Ta=25℃):

| Parameter | Symbol | Condition | Color | Rating | Units |
| --- | --- | --- | --- | --- | --- |
| Power Dissipation Segment/DP | Pd | ─ | Red | 250/50 | mW |
| Forward Current Segment | IF | ─ | Red | 25 | mA |
| Derating Of If Per Segment | △IF | Ta≧25℃ | Red | 0.30 | mA/℃ |
| Peak Forward Current Per Segment | IFP | 1/10 Duty 10KHz | Red | 100 | mA |
| Reverse Voltage Per Segment/DP | VR | ─ | Red | 25/5 | V |
| Operating Temperature Range | Topr | ─ | ─ | -35～+85 | ℃ |
| Storage Temperature Range | Tstg | ─ | ─ | -35～+85 | ℃ |

■Electrical/Optical Characteristics Rating(Ta=25℃):

| Item | Symbol | Test conditions | Location | Color | Rating |  |  | Units |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Min. | Typ. | Max. |  |
| Forward Voltage | VF | IF=20mA | ## Per Segment | ## Red | — | 9.5 | 10.5 | V |
|  |  |  | ## DP |  |  | 1.9 | 2.1 |  |
| Reverse Current | IR | VR=25V | Per Segment | Red | — | — | 100 | μA |
|  |  | VR=5V | DP |  |  |  |  |  |
| Luminous Intensity | IV | IF=10mA | Per Segment | Red | 15.0 | 20.0 | — | mcd |
| Wave Length | λP | IF=20mA | Per Segment | ## Red | — | 660 | — | nm |
|  | λD |  |  |  |  | 643 |  |  |
| Spectral Line Half Width | △λ | IF=20mA | Per Segment | ## Red | — | 20 | — | nm |
| Luminous Intensity Matching Ratio (Segment to Segment) | Iv-m | IF=10mA | ─ | ─ | ─ | ─ | 2:1 |  |

■Notes: Luminous intensity tolerance : +/-10%

■Soldering Conditions: Soldering Temp. ≦+260℃, Soldering Time. ≦3sec.

(at 2mm Distance from The Case of Reflector Edge)

3/3

WCN1-00D0SD-A11S