World Components Network Service Ltd

WCN1-00F0SD-A23

SPECIFICATION

| WCN |  |  | CUSTOMER |
| --- | --- | --- | --- |
| NOTE: |  |  |  |

REVISION:  02

1/3

World Components Network Service Ltd

- Outer Dimension:

Notes: Unless otherwise stated, The tolerance is ±0.25mm.

- Circuit Diagram:

- Pin Connection:

| PIN NO. | CONNECTION | PIN NO. | CONNECTION |
| --- | --- | --- | --- |
| 1 | Common Cathode | 6 | Anode B |
| 2 | Anode E | 7 | Anode A |
| 3 | Anode D | 8 | Common Cathode |
| 4 | Anode C | 9 | Anode F |
| 5 | ### Anode DP | 10 | Anode G |

2/3

WCN1-00F0SD-A23

World Components Network Service Ltd

- Features:

.  High Reliability

.  Color: Red

.  Low Power Requirement

.  Easy Assembly

- Description:

.  Single Digit LED Display

.  Digit Height:152.4mm(6.0〞)

.  Black Face And Encapsulation With Milky Segment

■Absolute Maximum Rating (Ta=25℃):

| Parameter | Symbol | Condition | Color | Rating | Units |
| --- | --- | --- | --- | --- | --- |
| Power Dissipation Per Segment/DP | Pd | ─ | Red | 540/180 | mW |
| Forward Current Per Segment | IF | ─ | Red | 45 | mA |
| Derating Of If Per Segment | △IF | Ta≧25℃ | Red | 0.30 | mA/℃ |
| Peak Forward Current Per Segment | IFP | 1/10 Duty 10KHz | Red | 100 | mA |
| Reverse Voltage Per Segment/DP | VR | ─ | Red | 30/10 | V |
| Operating Temperature Range | Topr | ─ | ─ | -35～+85 | ℃ |
| Storage Temperature Range | Tstg | ─ | ─ | -35～+85 | ℃ |

■Electrical/Optical Characteristics Rating(Ta=25℃)

| Item | Symbol | Test conditions | Location | # Color | Rating |  |  | Units |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Min. | Typ. | Max. |  |
| Forward Voltage | VF | IF=20mA | # Per Segment | Red | — | 11.4 | 12.6 | V |
|  |  |  | DP |  |  | 3.8 | 4.2 |  |
| Reverse Current | IR | VR=30V | Per Segment | Red | — | — | 100 | μA |
|  |  | VR=10V | DP |  |  |  |  |  |
| Luminous Intensity | IV | IF=10mA | Per Segment | Red | 10 | 15.0 | — | mcd |
| Wave Length | λP | IF=20mA | Per Segment | Red | — | 660 | — | nm |
|  | λD |  |  |  |  | 640 |  |  |
| Spectral Line Half Width | △λ | IF=20mA | Per Segment | Red | — | 20 | — | nm |
| Luminous Intensity Matching Ratio (Segment to Segment) | Iv-m | IF=10mA | ─ | ─ | ─ | ─ | 1.3:1 |  |

- Luminous Intensity Sorting: (Luminous Intensity Tolerance is +/-10%)

| Rank | Symbol | Condition | ## Min | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| O | O | IF=10mA | 25991 | 33790 | μcd |
| P | P | IF=10mA | 33791 | 43990 | μcd |
| Q | Q | IF=10mA | 43991 | 57100 | μcd |
| R | R | IF=10mA | 57101 | 74230 | μcd |

- Soldering Conditions: Soldering Temp. ≦+260℃

Soldering Time. ≦3sec. (at 2mm Distance from The Case of Reflector Edge)

3/3

## WCN1-00F0SD-A23