World Components Network Service Ltd

WCNCK-0033SR-A61

SPECIFICATION

| WCN |  |  | CUSTOMER Confirmed |
| --- | --- | --- | --- |
| Prepared by | Checked by | Approved by |  |

REVISION:  01

1/3

World Components Network Service Ltd

■ Outer Dimension:

Notes: Unless otherwise stated, The tolerance is ±0.25mm.

■ Circuit Diagram

■ Pin Connection:

| PIN NO. | CONNECTION | PIN NO. | CONNECTION |
| --- | --- | --- | --- |
| 1 | Cathode PM | 11 | Cathode G |
| 2 | Common Dig.1, AM | 12 | Common Dig.4 |
| 3 | Cathode D | 13 | Cathode B |
| 4 | Cathode C | 14 | Cathode A |
| 5 | Common LC,UC | 15 | ## Cathode F |
| 6 | Common Dig.3 | 16 | Cathode UC,LC |
| 7 | Anode DP | 17 | Common Dig.2 |
| 8 | Cathode DP | 18 | # Cathode AM |
| 9 | Anode ALARM | 19 | # Anode PM |
| 10 | Cathode ALARM | 20 | # Cathode AM |

2/3

WCNCK-0033SR-A61

World Components Network Service Ltd

■ Features:

.  High Reliability

.  Color: Red

.  Low Power Requirement

.  Easy Assembly

- Description:

.  Four Digit Clock Display

.  Digit Height: 8.38(0.33”)

.  Black Face and Milky Segment

■ Absolute Maximum Rating (Ta=25℃):

| Parameter | Symbol | Condition | Color | Rating | Units |
| --- | --- | --- | --- | --- | --- |
| ### Power Dissipation Per Segment | Pd | ─ | Red | 65 | mW |
| Forward Current Per Segment | IF | ─ | Red | 25 | mA |
| ### Derating Of If Per Segment | △IF | Ta≧25℃ | Red | 0.30 | mA/℃ |
| ### Peak Forward Current Per Segment | IFP | 1/10 Duty 10KHz | Red | 100 | mA |
| ### Reverse Voltage Per Segment | VR | ─ | Red | 5 | V |
| Operating Temperature Range | Topr | ─ | ─ | -35～+85 | ℃ |
| Storage Temperature Range | Tstg | ─ | ─ | -35～+85 | ℃ |

■ Electrical/Optical Characteristics Rating(Ta=25℃)

| Item | Symbol | Test conditions | Location | Rating |  |  | Units |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min. | Typ. | Max. |  |
| Forward Voltage | VF | IF=20mA | # Per Segment | — | 2.10 | 2.60 | V |
| Reverse Current | IR | VR=5V | Per Segment | — | — | 100 | μA |
| Luminous Intensity | IV | IF=10mA | Per Segment | 3201 | 6000 | — | μcd |
| Peak Emission Wave Length | λP | IF=20mA | Per Segment | — | 645 | — | nm |
|  | λD |  |  |  | 638 |  |  |
| Spectral Line Half Width | △λ | IF=20mA | Per Segment | — | 20 | — | nm |
| Luminous Intensity Matching Ratio (Segment to Segment) | Iv-m | IF=10mA | ─ | ─ | ─ | 2:1 |  |

■ Luminous Intensity Sorting: (Luminous Intensity Tolerance is +/-10%)

| Rank | Symbol | Condition | ## Min | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| J | J | IF=10mA | 3201 | 5050 | μcd |
| ## K | ## K | IF=10mA | 5051 | 8000 | μcd |
| L | L | IF=10mA | 8001 | 12650 | μcd |

■ Soldering Conditions: Soldering Temp. ≦+260℃, Soldering Time. ≦3sec.

(at 2mm Distance from The Case of Reflector Edge)

3/3

WCNCK-0033SR-A61